INTEGRATION SPECIFICATION

Pybricks Hub Protocol and Command Set

Shared BLE contract for iOS, Android, and the Raspberry Pi Python web app

| Status | Scope | Source baseline |
| --- | --- | --- |
| Draft v1.0 • 16 Aug 2026 | SPIKE Prime / Pybricks Wall‑E companion | Existing iOS, web bridge, and hub program |

# 1. Purpose and decision

This document defines the one app-to-hub contract that new clients must implement. It standardizes the newline-delimited Wall‑E companion protocol already used by the Python web app and hub program. The next iOS and Android releases should adopt this contract so every client sends the same commands, parses the same events, and preserves the same robot safety behavior.

Normative choice: use the current line protocol (READY / ACK / ERR / SAFE / TEL), not the older fixed three-byte iOS demo protocol. The older `fwd`, `rev`, `stp`, `rdy`, and `ack:` exchange is legacy-only and must not be used by new integrations.

# 2. Architecture and responsibilities

| Layer | Responsibility | Implementation rule |
| --- | --- | --- |
| Mobile or web client | Scan, select, connect, subscribe, issue supported commands, render status. | Never bypass hub safety rules or assume a command succeeded without an ACK/event. |
| BLE transport | Carries Pybricks command input and stdout notifications. | One active host connection to the hub at a time. |
| Hub program | Parses complete lines, drives motors, owns safety stops, emits telemetry. | It is the authority for obstacle, touch, timeout, and speed limits. |
| Optional web API | Exposes a deliberately limited UI-facing control surface. | Map app actions to allow-listed hub commands; do not expose arbitrary command input. |

# 3. BLE transport contract

| Item | Value / behavior |
| --- | --- |
| Advertising filter | Pybricks service UUID: c5f50001-8280-46da-89f4-6d8051e4aeef |
| Command/event characteristic | c5f50002-8280-46da-89f4-6d8051e4aeef |
| Client → hub packet | Write with response. Byte 0 is 0x06 (Pybricks WRITE_STDIN); remaining bytes are UTF-8 command text terminated by LF (0x0A). |
| Hub → client notification | First byte 0x01 means Pybricks WRITE_STDOUT. Treat the remaining bytes as a continuous UTF-8 byte stream. |
| Framing | Buffer stdout bytes until LF. A notification can contain part of a line, one line, or multiple lines. Do not assume notification boundaries are message boundaries. |
| Connection model | BLE permits one active client host. A new client must disconnect the existing app first. |

Command packet:  06 44 52 56 20 2D 33 35 20 2D 33 35 0A
                 |  D  R  V     -  3  5     -  3  5  LF
Notification:    01 52 45 41 44 59 0A   → stdout line: READY

# 4. Session lifecycle

Scan only for the Pybricks service UUID and show a user-selected hub name/address.

Connect, discover the command/event characteristic, and enable notifications.

Set UI state to Connected — start the Pybricks program. Do not enable motion controls yet.

When the `READY` line arrives, set ready=true and enable approved controls. `STOP` may remain available once connected.

Send one complete LF-terminated command per user action. Await ACK, ERR, SAFE, or timeout before presenting completion.

On disconnection, clear ready state, buffered stdout, pending command state, and motion UI state. Reconnect only after a fresh scan/selection or explicit reconnect policy.

# 5. Command set (client → hub)

Commands are uppercase ASCII tokens separated by one space and terminated by LF. New apps must only send the commands in this table. Percent values are signed integers; the hub clamps drive values to its configured maximum and head targets to its configured limit.

| Command | Arguments | Meaning | Expected hub response |
| --- | --- | --- | --- |
| STOP | none | Immediately stop both drive motors. | ACK STOP |
| PING | none | Liveness check; no movement. | ACK PING |
| DRV <left> <right> | integers, nominally −55…55 | Set left/right wheel power. Positive logical drive direction is defined by the hub’s motor-sign configuration. | ACK DRV <left> <right>; an obstacle stop emits SAFE obstacle before the ACK |
| HEAD <degrees> | integer, nominally −60…60 | Track the head to target angle. Hub clamps out-of-range targets. | ACK HEAD <clamped degrees> |

Recommended control mapping: forward = `DRV -35 -35`; reverse = `DRV 35 35`; left = `DRV 35 -35`; right = `DRV -35 35`; head left = `HEAD -45`; head right = `HEAD 45`; stop = `STOP`.

# 6. Event and response set (hub → client)

| Line | Meaning | Client action |
| --- | --- | --- |
| READY | Hub program has started and accepts commands. | Set ready=true; enable normal controls. |
| ACK <command> | A command was accepted and executed or scheduled. | Resolve the matching pending command and update the UI. |
| ERR invalid-drive / invalid-head | Argument could not be parsed. | Show a non-motion error; keep the connection alive. |
| ERR line-too-long | Input exceeded the hub’s line buffer. | Treat as a client bug; stop sending and log diagnostic context. |
| ERR <command> | Unsupported command. | Treat as an integration error; do not retry blindly. |
| SAFE obstacle / shoulder | Hub stopped the drive due to detected obstacle or touch switch. | Immediately show safety state; do not auto-resume movement. |
| TEL d=<mm> r=<0–100> touch=<0¦1> h=<degrees> | Periodic sensor/motion telemetry (currently every 500 ms). | Parse known key/value fields; ignore unknown future fields for forward compatibility. |

# 7. Safety and reliability requirements

The hub’s two-second command timeout is mandatory. Apps that need sustained movement must refresh `DRV` commands at a safe cadence (for example, every 250–500 ms) while the control is held; send `STOP` on release.

A `SAFE` event, disconnect, app backgrounding, BLE write failure, or control timeout must put the client UI into a stopped state. Never automatically replay the prior motion command after recovery.

Treat `ACK` as application-level confirmation. A successful BLE write only confirms transport delivery, not motor execution.

Use one in-flight motion command per client. Coalesce rapid joystick/button updates so a stale queued command cannot re-start motion after `STOP`.

Clients must not depend on telemetry delivery for safety: stdout is non-blocking and telemetry or ACK lines can be dropped if the hub’s buffer is full.

# 8. Platform implementation notes

| Platform | Required approach |
| --- | --- |
| iOS (CoreBluetooth) | Use the Pybricks UUIDs, write `Data([0x06]) + utf8(command + "\n")` with response, subscribe to the same characteristic, and line-buffer stdout after stripping event byte 0x01. Replace the current fixed-three-byte demo controller for Wall‑E mode. |
| Android (BluetoothGatt) | Scan by service UUID, request/connect GATT, discover services, enable characteristic notifications/CCCD, write the 0x06-prefixed LF-terminated payload with response, and line-buffer notification payload after 0x01. |
| Python web bridge (Bleak) | Maintain one async BLE client for the Flask app. Restrict HTTP actions to an allow-list that maps to this command set; keep raw command text off public endpoints. |
| Hub program (Pybricks) | Poll stdin, build a bounded LF-delimited line, emit documented text lines with `print`, and retain ownership of safety conditions and output cadence. |

# 9. Android reference pseudocode

onNotification(bytes):
  if bytes[0] != 0x01: return
  stdoutBuffer.append(bytes.drop(1))
  while stdoutBuffer contains LF:
    line = decodeUtf8(popLine(stdoutBuffer)).trim()
    handleHubLine(line)

send(command):
  require(connected && (ready || command == "STOP"))
  payload = [0x06] + utf8(command + "\n")
  writeCharacteristic(commandEvent, payload, WRITE_TYPE_DEFAULT)

# 10. Compatibility and versioning

The repository contains two different protocols. The `PybricksRemote` iOS demo uses an unframed, fixed three-byte exchange (`rdy`, `fwd`/`rev`/`stp`, `ack:`). The Wall‑E companion and Raspberry Pi web bridge use this document’s newline-delimited protocol. They are not interoperable without a translation layer.

Version this document and the hub program together. Add future capabilities as new uppercase commands/events; clients must ignore unknown `TEL` keys and display unknown event lines only as diagnostics.

If supporting the legacy demo is required, expose it as a separate explicit device/profile choice. Do not infer protocol mode from Bluetooth UUIDs because both variants use the same Pybricks transport service.

# 11. Acceptance checklist

Client discovers only advertised Pybricks-service hubs and handles Bluetooth permission/state errors.

Client correctly reconstructs `READY`, `ACK`, `SAFE`, `ERR`, and `TEL` lines across split or combined notifications.

Client sends exactly `0x06 + UTF-8(command) + LF` for all commands and uses write-with-response.

STOP, disconnect, backgrounding, write failure, and SAFE status leave the UI visibly stopped without auto-resume.

Sustained drive refreshes occur before the hub’s 2 s timeout; release sends STOP.

No API or UI permits arbitrary hub command injection; commands are allow-listed and logged safely.

# Appendix A — Reference sources in this workspace

Hub behavior: `walle_hub/walle_companion.py`. Web BLE bridge: `rpi_yolo_webapp/app.py`. Existing iOS fixed-length demo: `ios/PybricksRemote/PybricksRemote/PybricksHubController.swift`. This specification records the currently implemented Wall‑E protocol; it is not an official Pybricks protocol reference.